Computer Systems (CSIT-230) 	MONTCLAIR STATE UNIVERSITY 	 Spring 2025

______________________________________________________________________________________

Lab Assignment (Part of Discussion 10% of Overall Grade)  100 pts

Title: Logic Circuit Design and Implementation on a Solderless Breadboard

Group Size: 4 Students per Group

Overview:  This assignment consists of two parts:

Part 1: Report Submission

Each group will:

- Select a Logic Circuit: Choose one logic circuit from the topics covered in previous lectures. Examples include:

  - Basic Logic Gates (AND, OR, NOT, NAND, NOR, XOR, XNOR)

Boolean Expression Used:   F = AB

  - Half Adder or Full Adder

  - 7-Segment Display Decoder

  - Word Problems (e.g., designing a circuit for a specific real-world problem)

- Write a Detailed Report: The report should include the following sections:

  - Truth Table: Make the truth table of the selected logic circuit.

| A | B | F |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 0 |
| 1 | 0 | 0 |
| 1 | 1 | 1 |

  - Circuit Diagram: Provide a clear and labeled diagram of the circuit.

  - Equipment List: Specify the exact number and type of components required, such as:

    - 1 LEDs

    - 3 Resistors

    - 8 - 10 Wires (Dupont wires)

    - 7408 IC (AND gate)

    - 1 Battery/Power Supply

    - 2 Push Buttons

    - 1 Solderless Breadboard

- Breadboard Orientation and Working: Explain how a solderless breadboard works, including:

    - How current flows through the breadboard.

    - The orientation of power rails and terminal strips.

      - A solderless breadboard is a tool used to build and test electronic circuits without soldering. It has internal metal strips that create connections between holes, allowing current to flow through the board. The breadboard is divided into two main areas, the power rails and terminal strips. Power rails run vertically along the sides and are used to distribute power marked with + and –. Terminal strips are in the middle area, where there are horizontal rows of five. Allowing the components to be linked together. A central gap separates the terminal strips and is often used to place integrated circuits, ensuring each pin sits in a separate row. This layout makes it easy to prototype and modify circuits.

Implementation Steps: Provide a step-by-step guide to building your circuit on the breadboard.

  - Place the 7408 IC:

    - Insert the 7408 IC into the breadboard, ensuring each pin has its designated space.​

  - Power the IC:

    - Connect Pin 14 (VCC) to the positive 5V rail.​

    - Connect Pin 7 (GND) to the ground rail.​

  - Connect the Inputs:

    - Connect one terminal of each push-button switch to the 5V rail.​

    - Connect the other terminal of each switch to Pins 1 and 2 (inputs of the first AND gate) respectively.

  - Add Pull-Down Resistors:

    - Connect a 10 kΩ resistor from Pin 1 to the ground rail.​

    - Connect another 10 kΩ resistor from Pin 2 to the ground rail.​

  - Connect the Output:

    - Connect Pin 3 (output of the first AND gate) to the anode of the LED through a 220 Ω resistor.​

    - Connect the cathode of the LED to the ground rail.​

  - Test the Circuit:

    - Press both push-button switches simultaneously.​

    - The LED should light up only when both inputs are HIGH, indicating the AND gate is functioning correctly.​

  - References: Include links to resources used (e.g., Wikipedia, YouTube videos, textbooks).

    - https://www.instructables.com/Creating-an-AND-Gate-Circuit-Using-7408-IC/

    - https://www.youtube.com/watch?v=I0jgY0N4AMk&list=LL&index=1

Submit the Report: Every member of the group will submit the same report as a PDF file through Canvas.

Due Date for Part 1: 30th March 2025 - Sunday 11:59 PM

| Instructor: Dr. Prince Waqas Khan | 1 |
| --- | --- |

Computer Systems (CSIT-230) 	MONTCLAIR STATE UNIVERSITY 	 Spring 2025

______________________________________________________________________________________

Part 2: Practical Implementation

During the lab session:

- Build the Circuit: Using the equipment listed in your report, implement the selected logic circuit on a solderless breadboard.

- Demonstrate the Circuit: Show the working circuit to the professor or lab instructor.

- Answer Questions: Be prepared to explain your circuit design and answer any questions related to its functionality.

Date for Part 2: (2nd  April 2025- Wednesday- During class timing)

# Grading Criteria

The assignment will be graded based on the following criteria:

- Report (50%):

  - Clarity and completeness of the report and accuracy of the equipment list.

  - Explanation of breadboard working and implementation steps.

- Practical Implementation (50%):

  - Successful construction of the circuit on the breadboard.

  - Ability to explain the circuit and answer questions.

# Instructions for Group Formation

- Form a group of 4 students.

- Nominate a group leader.

- Only the group leader will submit the group’s information (names, email IDs, and selected circuit) via the Google Forms link provided in the description.

# Important Notes

- Equipment: You do not need to purchase any equipment. All required components will be provided during the lab session.

- Collaboration: While collaboration within the group is encouraged, each group must submit a unique circuit design. One circuit for one Group. Same report for all 4 members of the group.

# Resources

Here are some resources to help you get started:

- Breadboard Basics:

  - Wikipedia: Breadboard o 	YouTube: How to Use a Breadboard

- Logic Gates and Circuits:

  - YouTube: Logic Gate Tutorials

- Equipment List:

  - http://us.elegoo.com/products/elegoo-uno-r3-super-starterkit?srsltid=AfmBOoqLriGh3cBMawPsYMcJNIlFFcBMV-RMKlBGfA5j0NDU41zdflFg

|  | Good luck |  |
| --- | --- | --- |
| Instructor: Dr. Prince Waqas Khan |  | 2 |