## How does the glassware we use affect precision and accuracy of our measurements?
Duy Hoang
Experiment Date: 9/16/2025
Report Due Date: 9/23/2025

## Results and Discussion

The purpose of this lab was to examine how the choice of glassware influences the accuracy and precision of solution preparation. By comparing solutions made with a volumetric flask versus a beaker, we can see how the different tolerances in glassware affect the variability of measured concentrations.

Connection to Beer’s Law
The background absorbance at 412.80 nm was measured 5 times. Each raw absorbance was corrected by subtracting this blank. Concentration was determined using Beer’s Law:

    A = ε b c   →   c = A / (ε b)

where ε = 1020 M⁻¹ cm⁻¹ and b = 1.00 cm.

## Data Presentation and Analysis

Volumetric Flask Method

Stock solution: 2.0010 mM = 2.0010 × 10⁻³ M
Dilution: 25.00 mL transferred → 50.00 mL final (1:2 dilution)
Expected concentration:
    c = (2.0010 × 10⁻³)(25.00/50.00) = 1.0005 × 10⁻³ M

Uncertainty sources:
    • 25.00 mL pipette ±0.03 mL
    • 50.00 mL volumetric flask ±0.05 mL

Relative uncertainty:
    sqrt((0.03/25.00)² + (0.05/50.00)²) = 0.00156 (0.156%)

Absolute uncertainty:
    u_c = (1.0005 × 10⁻³)(0.00156) = 1.56 × 10⁻⁶ M

Final result:
    c = (1.0005 ± 0.0016) × 10⁻³ M

Beaker Method

Dilution: 25.0 mL transferred → 50.0 mL final (1:2 dilution)
Expected concentration:
    c = (2.0010 × 10⁻³)(25.0/50.0) = 1.0005 × 10⁻³ M

Uncertainty sources:
    • Beaker ±5% on both transfer and final volume

Relative uncertainty:
    sqrt((0.05)² + (0.05)²) = 0.0707 (7.07%)

Absolute uncertainty:
    u_c = (1.0005 × 10⁻³)(0.0707) = 7.08 × 10⁻⁵ M

Final result:
    c = (1.0005 ± 0.0708) × 10⁻³ M

### Table 1. Variability of potassium ferricyanide concentration for each replicate

| Sample | Ave. conc. (M) | SD(M) | RSD(%) | 95%Cl(M) |
| --- | --- | --- | --- | --- |
| 1 | 9.22×10⁻⁴ | 7.19×10⁻⁶ | 0.753 | 6.30×10⁻⁶. |
| 2 | 9.87×10⁻⁴ | 3.16×10⁻⁵ | 3.20 | 2.77×10⁻⁵ |
| 3 | 9.78×10⁻⁴ | 9.21×10⁻⁶ | 0.941 | 8.07×10⁻⁶ |
| 4 | 9.19×10⁻⁴ | 1.04×10⁻⁵ | 1.13 | 9.12×10⁻⁶ |
| 5 | 9.92×10⁻⁴ | 1.41×10⁻⁵ | 1.42 | 1.23×10⁻⁵ |
| 6 | 9.76×10⁻⁴ | 4.34×10⁻⁶ | 0.444 | 3.80×10⁻⁶ |

Table 1 shows the average concentration, standard deviation (SD), relative standard deviation (RSD), and 95% confidence interval (CI) for each of the six solutions. This table demonstrates the spread of the measurements within each solution.

### Table 2. Variability and error by preparation method

| Method | Avg conc.(M) | SD(M) | RSD(%) | 95% Cl(M) | Abs. Error (M) | % Error |
| --- | --- | --- | --- | --- | --- | --- |
| Flask | 9.74×10⁻⁴ | 2.29×10⁻⁵ | 2.35 | 1.16×10⁻⁵ | 2.70×10⁻⁵ | 2.7 |
| Beaker | 9.62×10⁻⁴ | 3.36×10⁻⁵ | 3.49 | 1.70×10⁻⁵ | 3.82×10⁻⁵ | 3.8 |

Table 2 combines the data from Table 1 into group statistics for flask vs. beaker preparations. It summarizes the average concentration, SD, RSD, CI, and also introduces absolute error and percent error relative to the expected theoretical concentration (1.0005×10⁻³ M).

## Discussion Questions

1. Which preparation method gives the most accurate result?
The volumetric flask preparation was more accurate, giving an average concentration closer to the expected 1.0005 × 10⁻³ M and a smaller % error (2.7% vs 3.8%).

2. Case 1 t-test: Does either method agree with the true value?
t = |x̄ – μ| / (s / √n)

From the results (Table 2):
• Volumetric flask mean: 9.74×10⁻⁴ M
• Beaker mean: 9.62×10⁻⁴ M
• True value: 1.0005×10⁻³ M
• 95% confidence intervals:
   – Flask: (9.63–9.85)×10⁻⁴
   – Beaker: (9.45–9.79)×10⁻⁴

The true value (1.0005×10⁻³ M) lies outside both confidence intervals. Therefore neither method agrees with the true value at the 95% confidence level. Even though the flask method is more accurate, both methods underestimate the concentration slightly, and their error is large enough that we reject the hypothesis that they equal the true value

3. Which method appears to be more precise?
The volumetric flask method is more precise, as shown by a lower standard deviation, smaller RSD, and narrower confidence interval.

4. Case 2 t-test: Do the two means agree within experimental error?
t = |x̄₁ – x̄₂| / (s_p √(2/n))

From the results (Table 2):
• Flask mean: 9.74×10⁻⁴ M
• Beaker mean: 9.62×10⁻⁴ M
• Difference: 1.2×10⁻⁵ M
• SDs: 2.29×10⁻⁵ (flask), 3.36×10⁻⁵ (beaker)

The average concentration from the volumetric flask method (9.74×10⁻⁴ M) and the beaker method (9.62×10⁻⁴ M) are very close, with only a small difference of 1.2×10⁻⁵ M. Comparing their standard deviations, this difference is too small to be considered meaningful. A Case 2 t-test confirms that the two means are not statistically different at the 95% confidence level.