# Spectrophotometric Determination of the pKₐ of an Acid–Base Indicator

Student

Institution

Course

Professor

Date

# Spectrophotometric Determination of the pKₐ of an Acid–Base Indicator

We measured absorbance for the base form (In⁻) and acid form (HIn) of bromocresol green (BCG) to determine the indicator pKₐ. The λmax of the base form was used as the analytical wavelength for all measurements. A set of HOAc/NaOAc buffers (Flasks 3–7) spanning pH 3.8–5.8 was prepared. For each buffer, we recorded the absorbance AS at λmax(In⁻), then computed the distribution ratio log10In-HIn, using anchor absorbances from the acid-only and base-only solutions. A linear fit of log10In-HInvs pH yields the indicator pKₐ from the intercept (ideal slope ≈ 1).

### Materials & Key Settings

- Indicator: BCG

- Analytical wavelengths: λmax(In⁻) = 612.90 nm (also noted once as 617.90 nm; 612.90 nm used), λmax(HIn) = 439.20 nm

- Anchor absorbances at λmax(In⁻): A_B (base) = 1.556, A_A (acid) = 0.029

- Buffers: 1.00 M HOAc / 1.00 M NaOAc; each flask diluted to 50.00 mL

### Equations

- Delivered volumes (from buret readings): Vdelivered​=Vf​-Vi​

- Buffer concentrations after dilution to 50.00 mL:

[HOAc]=1.00 M⋅VHOAc50.00 mL

[NaOAc]=1.00 M⋅VNaOAc​​50.00 mL

- Henderson–Hasselbalch (pH of each buffer) using pKₐ(HOAc) = 4.76:

pH=pKa​+ log10In-HIn

- Indicator distribution from absorbances (all at λmax(In⁻)):

In-HIn=AS​-AA​​AB​-AS​​

y≡log10In-HIn

- Linear model for pKₐ of the indicator:

y=m(pH)+b ⇒ (ideal) m≈1,pKa​(indicator)=-b

### Data & Calculations

Buret readings (Vi → Vf), delivered volumes, concentrations, and pH

| Flask | HOAc Vi (mL) | HOAc Vf (mL) | HOAc deliv (mL) | NaOAc Vi (mL) | NaOAc Vf (mL) | NaOAc deliv (mL) | [HOAc] (M) | [NaOAc] (M) | pH |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | 10.21 | 33.19 | 22.98 | 24.08 | 26.61 | 2.53 | 0.4596 | 0.0506 | 3.8018 |
| 4 | 8.31 | 27.31 | 19.00 | 26.61 | 33.04 | 6.43 | 0.3800 | 0.1286 | 4.2895 |
| 5 | 27.31 | 39.89 | 12.58 | 33.04 | 45.82 | 12.78 | 0.2516 | 0.2556 | 4.7669 |
| 6 | 34.89 | 45.88 | 10.99 | 18.56 | 37.53 | 18.97 | 0.2198 | 0.3794 | 4.9971 |
| 7 | 45.88 | 47.85 | 1.97 | 15.28 | 38.30 | 23.02 | 0.0394 | 0.4604 | 5.8276 |

Absorbances at λmax(In⁻):
Flasks 3–7: AS = [0.106, 0.383, 0.849, 1.165, 1.480]
Anchors: AB = 1.556, AA = 0.029

Indicator ratio and log-ratio (per flask)

Ratio=A​S-AAAB​-AS

| Flask | Ratio | log 10 (Ratio) |
| --- | --- | --- |
| 3 | 0.05310 | −1.27488 |
| 4 | 0.30179 | −0.52029 |
| 5 | 1.15983 | 0.06439 |
| 6 | 2.90537 | 0.46320 |
| 7 | 19.09211 | 1.28085 |

Linear fit y=m⋅pH+by In the five data points:

- Slope, mmm = 1.262

- Intercept, bbb = −5.975

- R2R^2R2 = 0.9896

- Estimated pKₐ(BCG) = −b = 5.975

Ideal theory gives m≈1. The slope m=1.26m is close but >1, suggesting minor wavelength drift or an anchor absorbance slightly off at the chosen λ.

Graphs

Figure 1: Buret Readings (Vf)

Figure 2: First Derivative

Figure 3: Second Derivative

## 5.1  Flasks 1 and 2 - Extremes of pH

  - Color: Flask 1 = dark blue; Flask 2 = orange.

  - pH: Flask 1 (base) pH ≈ 11.00; Flask 2 (acid) pH ≈ 3.00.

  - Indicator form: Flask 1 = In− (basic form, blue); Flask 2 = HIn (acid form, orange).

  - λmax values: HIn ≈ 439.20 nm; In− ≈ 612.90 nm (used).

### Equation Work for pH

Each extreme flask: add 25.00 mL of 0.0020 M strong acid/base, dilute to 50.00 mL.
Final concentration C = 0.0020 M × 25.0050 = 0.0010 M.
Flask 1 (base): [OH−]= 0.0010 M ⇒ pOH = −log10(0.0010) = 3.00 ⇒ pH = 14.00 − 3.00 = 11.00.
Flask 2 (acid): [H+] = 0.0010 M ⇒ pH = −log10(0.0010) = 3.00.

### Table - Extremes of pH

| Flask | Color | pH | Form of indicator | λmax (nm) | Absorbance at λmax(In−) |
| --- | --- | --- | --- | --- | --- |
| 1 | Dark blue | ≈11.00 | In− (basic form) | 612.90 | 1.556 |
| 2 | Orange | ≈3.00 | HIn (acid form) | 439.20 | 0.029 |

### Discussion

The relationship between Log10In-HIn and pH was strongly linear across the buffer series (Flasks 3–7), with R² ≈ 0.99, indicating that the indicator behaved consistently with the Henderson–Hasselbalch model under our conditions. Using the anchor absorbances measured at the base-form wavelength, AB=1.556 and A​A=0.029, the fitted line yielded a slope m≈1.26 and intercept b≈-5.98, giving an experimental pKa=-b≈5.98 In the ideal two-species model, the slope is expected to be ~1 because Log10In-HIn=pH-pKa​. The modest deviation (m>1) suggests small, systematic effects rather than random noise.

Two factors likely dominate: wavelength anchoring and baseline/stray absorbance. The base-form λmax​ was noted as 612.90 nm in one place and 617.90 nm elsewhere. Reading all solutions (anchors and buffers) at precisely the same λmax is critical: a small wavelength offset changes the relative molar absorptivities of the acid and base forms and biases the ratio A​S-AAAB​-AS,, inflating the slope and shifting the intercept. Likewise, if the zero/baseline was not perfectly set, or if there was minor light scatter (fingerprints, bubbles, cuvette orientation), the finite absorbance window for the acid-rich solutions (near A≈AA​​) is especially susceptible to proportional error. These have systematic impacts on bending the regression downward of the optimal unit slope.

Preparation of the buffer adds more smaller uncertainties. The buret values (Vi→Vf) were transformed to received volumes and diluted to 50.00 mL; any error in delivery or meniscus value will be converted into small changes in [A⁻]/[HA] and subsequently in pH. Temperature and ionic strength are also important: the Henderson-Hasselbalch equation makes the assumption that activities are close to concentrations; deviations (especially in the higher concentrations of acetate) can change the apparent pKa.  Finally, the Beer–Lambert regime assumes linear absorbance vs concentration at the chosen indicator concentration; inner-filter or aggregation effects at higher absorbance (blue/base-rich flasks) can tilt the slope upward.

Despite these caveats, the data quality is high (tight monotonic trend, R2 near unity), and the qualitative color changes across the series match the indicator transition. The most direct, low-effort improvement would be to reconfirm λmax​ for the base form, re-read AB and AA​ at that exact wavelength, ensure the instrument zero is stable (baseline noise < ~0.01 A), and re-collect AS​ for the five buffers without changing wavelength. This typically brings m→1.00m and refines pKa downward toward commonly reported values for bromocresol green in water at 25 °C. Including uncertainty analysis (propagating baseline noise into the ratio and the regression) would add a defensible 95% CI to the reported pKa​.

In summary, the experiment successfully demonstrated the spectrophotometric determination of an indicator’s pKa​. The high linearity confirms the procedure, and the slope is larger than 1, which is in agreement with weak systematics (wavelength anchoring and baseline) that can be corrected. With such refinements, further improvement in precision and accuracy of the final pKa will be obtained.

“The results obtained using spectrophotometry at the λmax of base forms indicated a high linear correlation between log10In-HIn,  and pH (R² ≈ 0.99), which indicated Henderson-Hasselbalch behavior. The linear fit provided experimental pKa = 5.98, and the slope was less than one, suggesting some slight systematic influences. Tight wavelength control and baseline checks should provide a slope nearer to 1.00, and also an increase in the pKa accuracy/precision.”